Getting to know your VGA card

by Dave Coffin  11/21/93

The VGA memory inhabits the CPU address space from A0000

up to BFFFF.  Each address points to a row of eight pixels,

starting in the upper left corner.  In 640x480 mode the offsets

from segment A000h would be (in decimal):

000 001 002 003 004 005 006 ... 079

080 081 082 083 084 085 086 ... 159

160 161 162 163 164 165 166 ... 239

240 ...

From the CPU's point of view, each address holds eight bits

that can be read or written.  But it really points to a 32-bit

word in the VGA RAM (8 pixels, 4 bits each).  The graphics reg-

isters, accessed via I/O ports, specify how eight bits are trans-

lated into 32.  To write to a VGA register:

MOV DX,port	;I/O port number (usually shared)

MOV AL,index	;Choose a register

MOV AH,value	;Value to write

OUT DX,AX

On the EGA, these registers are all write-only.  On the VGA,

they may be read.

CPU data lines

|  |  |  |  |  |  |  |

V  V  V  V  V  V  V  V

Rotate by DR -> -> -> -> -> -> -> ->

Pixels -->  7  6  5  4  3  2  1  0     4-bit registers

Bit 	3   X  X  X  X  X  X  X  X  -- M  E        C  <-- CPU data lines

Planes	2   X  X  X  X  X  X  X  X  -- M  S  S  C  D  <-- using

1   X  X  X  X  X  X  X  X  -- R  R  R  C  C  <-- Write Mode 2

0   X  X  X  X  X  X  X  X  --                <--

|   |  |  |  |  |  |  |  |

RMS    Bit Mask Register

DX=3C4  AL=2  Map Mask Register (MMR)  4 bits

Selects which bitplanes may be modified by a

write operation.  Zero bits make the corresponding

bitplanes unwriteable.

DX=3CE  AL=8  Bit Mask Register  8 bits

Selects which pixels may be modified by a write

operation.  Zero bits make the corresponding pixels

unwriteable.

DX=3CE  AL=1  Enable Set/Reset (ESR)  4 bits

DX=3CE  AL=0  Set/Reset         (SR)  4 bits

If ESR is zero (the default), SR is ignored and

all the bitplanes are set from the data bus. If ESR

is all ones, each pixel is set to the color in SR.

DX=3CE  AL=4  Read Map Select (RMS)  2 bits

When the CPU reads a byte in mode 0 (the default),

this register selects which bitplane will be returned.

DX=3CE  AL=2  Color Compare (CC)     4 bits

DX=3CE  AL=7  Color Don't Care (CDC) 4 bits

When the CPU reads a byte in mode 1, it receives

a bitmask showing which pixels match the CC Register.

One bits in the CDC register cause the corresponding

bitplanes to be ignored during comparison.

DX=3CE  AL=5  Mode

AH=8 sets the read mode to 1 (see above).

The two low bits set the write mode:

AH=0: The 8-bit CPU data is copied to all bitplanes or

the 4-bit color in SR is copied to all pixels

AH=1: 32-bit data is copied unchanged from one video

address to another.

AH=2: 4-bit color from CPU is copied to all pixels.

AH=3: The CPU data specifies which pixels will be set

to the color in SR.  VGA only.

DX=3CE  AL=3  Data Rotate

Bits 0-3 specify the rotation to perform on incoming

data.  Bits 4 and 5 set the Function Select field.  During

a write operation, the "X" bits are modified as follows:

AH= 0: New data overwrites the old (default)

AH=16: New data is AND'ed with old data.

AH=32: New data is  OR'ed with old data.

AH=48: New data is XOR'ed with old data.